HDMI USB 16*1	SWITCH

USER MANUAL

1. Description

HDMI KVM switch sixteen groups of high-definition Audio and video source and USB interface implementation free switch, Adaptive EDID , can adapt to a variety of complex application environments.The equipment has excellent image processing and transmission capacity, make the output signal is more smooth and steady, is a reliable, high performance and efficient way of HDMI USB switch

- Features

- The sixteen-way HD Audio and video source choose USB interface to realize sixteen free switch;

- Support HDMI 1.4 standard protocol;

- Support video resolution max up to 4Kx2K@30Hz；

- Support USB 2.0 standard protocol;

- Adaptive EDID , able to adapt to a variety of complex application environment;

- Support Hot key switch, convenient and quick(ctrl + ctrl + 1,2,3……14,15,16 + Enter)

- High compatibility, can auto-match source and display device;

- Built-in automatic adjustment system, make the image smooth, clear and stable;

- Simple to install, plug and play;

3. Package list

        - HDMI KVM16*1Switcher host  		1	pcs

        - USB Cable    						16 	pcs

        - HDMI Cable                 		16 	pcs

        - Hardware accessories             2  	pcs

        - Power adapter                 	1  	pcs

        - user manual				      	1  	pcs

        - Warranty Card						1 	pcs

        - Foot Pad                     .  8  	pcs

4. Specifications

| Parameter |  | Description |
| --- | --- | --- |
| Video | Standards | HDMI 1.4 |
|  | Maximum pixel clock | 225MHz |
|  | Maximum data rate | 10.2Gbps |
|  | Resolution range | Up to 4Kx2K@30Hz, 3D, 1920*1080@60Hz |
|  | Connector | HDMI-A Interface |
|  | Impedance | 100Ω |
|  | HDMI Cable Range | <5M( Resolution: 1920*1080@60Hz) |
| USB | Interface protocols | USB1.1(2 pcs), USB 2.0(2 pcs) |
|  | PC input interface type | USB-B |
|  | Output interface type | USB-A |
| Other | Power supply | The power adapter: DC 12V |
|  | Power dissipation | MAX 5W |
|  | Temperature | Operating: -5℃ ~ +70℃ |
|  | Humidity | Operating: 5% ~ 90% |
|  | Dimension | 445*95*42mm |
|  | The warranty | 1 year free warranty |

5.Panel

| Index | Description |
| --- | --- |
| 1 | Power switch |
| 2 | RS232 interface |
| 3 | HDMI video output signal |
| 4 | PC1, PC2，PC3，PC4，PC5，PC6，PC7，PC8，PC9, PC10，PC11，PC12，PC13，PC14，PC15，PC16 HDMI and USB signal inputs |
| 5 | 12V power supply input |
| 6 | Data update interface |
| 7 | USB keyboard mouse interface |
| 8 | USB2.0 devices interface |
| 9 | PC1, PC2，PC3，PC4，PC5，PC6，PC7，PC8，PC9, PC10，PC11，PC12，PC13，PC14，PC15，PC16 switch buttons and status indicators |

6.Diagram

7.Serial command

Enter PS 1 to switch to port 1 (original keyboard command ctrl + ctrl +1)

Enter PS 2 to switch to port 2 (original keyboard command ctrl + ctrl + 2)

Enter PS 3 and switch to port 3 (original keyboard command ctrl + ctrl + 3)

Enter PS 6 to switch to port 4 (original keyboard command ctrl + ctrl + 6)

Enter PS 7 to switch to port 15 (original keyboard command ctrl + ctrl + 7)

Enter PS 8 to switch to port 16 (original keyboard command ctrl + ctrl + 8)

Enter CS 0 to stop scanning (no keyboard command, any key)

Enter CS 1 to start scanning (original keyboard command ctrl + ctrl +S)

You need to add ENTER at the end, and there must be a space before the number (except for hexadecimal commands)

8. Warranty

Please keep the user manual in a place where safety and easy to find so that you can read it immediately when meet problems.

safety instruction:

- Please do not disassemble this product by yourself because it doesn't including DIY function.

- Please don't make the product touch water in case damaged, because it is Not waterproof design.

- Please use dry cloth material when clean the product surface.

- Please do not use this product in the environment of too wet or too high temperature.

- Please avoid contact with corrosive substances.

- When there is damage or other physical damage to the product, please stop using and contact with customer service to get support.