HDMI KVM 4*1	SWITCH

USER MANUAL

1. Description

HDMI KVM switch four groups of high-definition Audio and video source and USB interface implementation free switch, Adaptive EDID , can adapt to a variety of complex application environments.The equipment has excellent image processing and transmission capacity, make the output signal is more smooth and steady, is a reliable, high performance and efficient way of HDMI USB switch.

- Features

- 4 ports HDMI KVM switch,4CH HDMI and USB HID signals synchronize switch;

- Supports video resolution max up to 4Kx2K@30Hz；

- Compliance with HDMI 1.4 standard;

- Adaptive EDID , able to adapt to a variety of complex application environment;

- Support hot key switch (Scroll Lock + Scroll Lock + 1,2,3,4) or (Ctrl + Ctrl + 1,2,3,4).

- When the Ctrl commands conflict, you can enter (Ctrl + Ctrl + C + T + R + L + 0 ) Disable, to re-enable the Ctrl hotkey command, you need to enter (Scroll Lock + Scroll Lock + C + T + R + L + 1).High compatibility, can auto-match source and display device.

- High compatibility, can auto-match source and display device;

- Built-in automatic adjustment system, make the image smooth, clear and stable;

- Easy to install, plug and play.

3. Package list

1 pcs

4 pcs

4 pcs

1 pcs

        - HDMI KVM4*1Switcher host

        - USB Cable

        - HDMI Cable

        - User manual

4. Specifications

| Parameter |  | Description |
| --- | --- | --- |
| Video | Standards | HDMI 1.4 |
|  | Maximum pixel clock | 225MHz |
|  | Maximum data rate | 10.2Gbps |
|  | Resolution range | Up to 4Kx2K@30Hz, 3D, 1920*1080@60Hz |
|  | Connector | HDMI-A Interface |
|  | Impedance | 100Ω |
|  | HDMI Cable Range | <5M( Resolution: 1920*1080@60Hz) |
| USB | Interface protocols | USB1.1 |
|  | PC input interface type | USB-B |
|  | Output interface type | USB-A |
| Other | Power supply | USB 5V power supply |
|  | Power dissipation | MAX 3W |
|  | Temperature | Operating: -5℃ ~ +70℃ |
|  | Humidity | Operating: 5% ~ 90% |
|  | Dimension | 230*74*25mm |
|  | The warranty | 1 year free warranty |

5. Panel

| Port name | Description |
| --- | --- |
| 1 | HDMI video signal output interface |
| 2~5 | PC1 , PC2，PC3，PC4 groups HDMI USB input |
| 6~9 | PC1 , PC2，PC3，PC4 groups buttons and status indicators |
| 10 | Power indicators |
| 11 | USB keyboard mouse connectors |

6.Diagram

7. Warranty

Please keep the user manual in a place where safety and easy to find so that you can read it immediately when meet problems.

safety instruction:

- Please do not disassemble this product by yourself because it doesn't including DIY function.

- Please don't make the product touch water in case damaged, because it is Not waterproof design.

- Please use dry cloth material when clean the product surface.

- Please do not use this product in the environment of too wet or too high temperature.

- Please avoid contact with corrosive substances.

- When there is damage or other physical damage to the product, please stop using and contact with customer service to get support.