HDMI USB 2*1	SWITCH

USER MANUAL

1. Description

HDMI KVM switch two groups of high-definition Audio and video source and USB interface implementation free switch, Adaptive EDID , can adapt to a variety of complex application environments.The equipment has excellent image processing and transmission capacity, make the output signal more smooth and steady, it is a reliable, high performance and efficient way of HDMI USB switch.

2. Features

- 2 ports KVM switch HDMI,2CH HDMI and USB HID signals synchronize switch.

- Support HDMI 2.0, HDCP2.2 standards.

- Support USB1.1 and USB2.0 protocol.

- Support hot key switch (Scroll Lock + Scroll Lock + 1, 2) or (Ctrl + Ctrl + 1, 2).

- When the Ctrl commands conflict, you can enter (Ctrl + Ctrl + C + T + R + L + 0 ) Disable, to re-enable the Ctrl hotkey command, you need to enter (Scroll Lock + Scroll Lock + C + T + R + L + 1).High compatibility, can auto-match source and display device.

- Support video resolution max up to 3840*2160@60Hz.

- Adaptive EDID , able to adapt to a variety of complex application environment.

- Built-in automatic adjustment system, make the image smooth, clear and stable.

- Simple to install, plug and play.

- Package list

| HDMI KVM2*1 Switcher host | 1 pcs |
| --- | --- |
| USB A-B Cable | 2 pcs |
| HDMI 2.0 Cable | 2 pcs |
| Micro USB Cable | 1 pcs |
| User manual+Warranty Card | 1 set |
| Foot Pad | 4 pcs |

4. Specifications

| Parameter |  | Description |
| --- | --- | --- |
| Video | Standards | HDMI 2.0 |
|  | Maximum pixel clock | 340MHz |
|  | Maximum data rate | 18Gbps |
|  | Maximum resolution | 3840*2160@60Hz, Backward compatible |
|  | Connector | HDMI-A Interface |
|  | Impedance | 100Ω |
|  | HDMI Cable Range | <5M( Resolution: 1920*1080@60Hz) |
| USB | Interface protocols | USB2.0+USB1.1 |
|  | PC input interface type | USB-B |
|  | Output interface type | USB-A |
| Other | Power supply | 5V power supply |
|  | Power dissipation | MAX 2W |
|  | Temperature | Operating: -5℃ ~ +70℃ |
|  | Humidity | Operating: 5% ~ 90% |
|  | Dimension | 130*76*26.5mm |
|  | The warranty | 1 year free warranty |

5. Panel

| Port name | Description |
| --- | --- |
| 1 | 5V power supply input port |
| 2 | HDMI video output signal |
| 3、4 | PC2、PC1 group HDMI USB input |
| 5 | PC1, PC2 switch buttons |
| 6、7 | PC1, PC2 status indicators |
| 8 | Power indicators |
| 9 | Keyboard & Mouse、USB2.0 device input |

6.Diagram

7. Warranty

Please keep the user manual in a place where safety and easy to find so that you can read it immediately when meet problems。

safety instruction:

- Please do not disassemble this product by yourself because it doesn't including DIY function.

- Please don't make the product touch water in case damaged,because it is Not waterproof design.

- Please use dry cloth material when clean the product surface.

- Please do not use this product in the environment of too wet or too high temperature.

- Please avoid contact with corrosive substances.

- When there is damage or other physical damage to the product,please stop using and contact with customer service to get support.