HDMI KVM 4*1	SWITCH

USER MANUAL

1. Description

HDMI KVM switch four groups of high-definition Audio and video source and USB interface implementation free switch, Adaptive EDID, can adapt to a variety of complex application environments. The equipment has excellent image processing and transmission capacity, make the output signal more smooth and steady, it is a reliable, high performance and efficient way of HDMI USB switch.

- Features

- 4 ports KVM switch HDMI, 4CH HDMI and USB HID signals synchronize switch.

- Support HDMI 2.0, HDCP 2.2 standards.

- Supports video resolution max up to 3840*2160@60Hz.

- Support HDMI dual display (this function is only available on computers that support dual display).

- Adaptive EDID , able to adapt to a variety of complex application environment.

- Support hot key switch(Scroll Lock + Scroll Lock + 1、2、3、4) or (Ctrl +Ctrl + 1、2、3、4).

- When the Ctrl commands conflict, you can enter (Ctrl + Ctrl + C + T + R + L + 0 ) Disable, to re-enable the Ctrl hotkey command, you need to enter (Scroll Lock + Scroll Lock + C + T + R + L + 1). High compatibility, can auto-match source and display device.

- High compatibility, can auto-match source and display device.

- Built-in automatic adjustment system, make the image smooth, clear and stable.

- Easy to install, plug and play.

3. Package list

| HDMI KVM 4*1 Switcher Host | 1 Pc |
| --- | --- |
| USB A-B Cable | 4 Pcs |
| HDMI Cable | 8 Pcs |
| Micro USB Cable | 1 Pc |
| User Manual+Warranty Card | 1 Pc |
| Foot Pad | 4 Pcs |

4. Specifications

| Parameter |  | Description |
| --- | --- | --- |
| Video | Standards | HDMI 2.0 |
|  | Maximum pixel clock | 340MHz |
|  | Maximum data rate | 18Gbps |
|  | Maximum resolution | 3840*2160@60Hz, Backward compatible |
|  | Connector | HDMI-A Interface |
|  | Impedance | 100Ω |
|  | HDMI Cable Range | <5M( Resolution: 1920*1080@60Hz) |
| USB | Interface protocols | USB1.+USB2.0 |
|  | PC input interface type | USB-B |
|  | Output interface type | USB-A |
| Other | Power supply | 5V power supply |
|  | Power dissipation | MAX 2.5W |
|  | Temperature | Operating: -5℃ ~ +70℃ |
|  | Humidity | Operating: 5% ~ 90% |
|  | Dimension | 202*76*27mm |
|  | The warranty | 1 year free warranty |

- Panel

| Port name | Description |
| --- | --- |
| 1 | 5V power supply , for high power USB device |
| 2 | Machine power switch |
| 3 | HDMI video output |
| 4~7 | PC1 , PC2，PC3，PC4 HDMI USB input |
| 8~11 | PC1 , PC2，PC3，PC4 buttons and status indicators |
| 12 | Power indicator |
| 13 | Keyboard & Mouse |

- Diagram

- Warranty

1. Connect HDMI/DVI/VGA/DP/Type-C to HDMI cables and USB-B cable between your computer 1 to 4 and "PC1 to PC4" on KVM.

2. Connect the HDMI output port on the KVM to your monitors with HDMI to HDMI/DVI/VGA/DP cables.

3. Connect the mouse and keyboard to the "K\M" port.

4. Turn on the power switch.

- Warranty

Please keep the user manual in a place where safety and easy to find so that you can read it immediately when meet problems.

safety instruction:

- Please do not disassemble this product by yourself because it doesn't including DIY function.

- Please don't make the product touch water in case damaged, because it is Not waterproof design.

- Please use dry cloth material when clean the product surface.

- Please do not use this product in the environment of too wet or too high temperature.

- Please avoid contact with corrosive substances.

- When there is damage or other physical damage to the product, please stop using and contact with customer service to get support.