HDMI USB 8*1	SWITCH

USER MANUA

1. Description

HDMI KVM switch eight groups of high-definition Audio and video source and USB interface implementation free switch, Adaptive EDID, can adapt to a variety of complex application environments. The equipment has excellent image processing and transmission capacity, make the output signal is more smooth and steady, is a reliable, high performance and efficient way of HDMI USB switch.

- Features

- The 8 ports HD Audio and video source choose USB interface to realize eight free switch;

- Supports video resolution max up to 4Kx2K@30Hz；

- Compliance with HDMI 1.4 standard;

- Support USB2.0 standard;

- Support hot key switch (Scroll Lock + Scroll Lock + 1,2,3,4,5,6,7,8) or (Ctrl + Ctrl + 1,2,3,4,5,6,7,8).

- When the Ctrl commands conflict, you can enter (Ctrl + Ctrl + C + T + R + L + 0 ) Disable, to re-enable the Ctrl hotkey command, you need to enter (Scroll Lock + Scroll Lock + C + T + R + L + 1).High compatibility, can auto-match source and display device.

- Adaptive EDID , able to adapt to a variety of complex application environment;

- High compatibility, can auto-match source and display device;

- Built-in automatic adjustment system, make the image smooth, clear and stable;

- Easy to install, plug and play;

3. Package list

| HDMI KVM8*1Switcher host | 1 pcs | USB-A to USB-B Cable | 8 pcs |
| --- | --- | --- | --- |
| HDMI Cable | 8 pcs | Hardware accessories | 2 pcs |
| RS232 terminal | 1 pcs | Power adapter | 1 pcs |
| User manual | 1 pcs | Warranty Card | 1 pcs |
| Foot Pad | 4 pcs |  |  |

4. Specifications

| Parameter |  | Description |
| --- | --- | --- |
| Video | Standards | HDMI 1.4 |
|  | Maximum pixel clock | 225MHz |
|  | Maximum data rate | 10.2Gbps |
|  | Resolution range | Up to 4Kx2K@30Hz, 3D |
|  | Connector | HDMI-A Interface |
|  | Impedance | 100Ω |
|  | HDMI Cable Range | <5M( Resolution: 1920*1080@60Hz) |
| USB | Interface protocols | USB1.1（2pcs）,USB2.0（2pcs） |
|  | PC input interface type | USB-B |
|  | Output interface type | USB-A |
| Other | Power supply | The power adapter: DC 12V |
|  | Power dissipation | MAX 5.5W |
|  | Temperature | Operating: -5℃ ~ +70℃ |
|  | Humidity | Operating: 5% ~ 90% |
|  | Dimension | 202*68*36mm |
|  | The warranty | 1 year free warranty |

5.Panel

| Port name | Description |
| --- | --- |
| 1 | 12V power supply input port |
| 2 | HDMI video signal output |
| 3 | RS232 interface |
| 4-11 | PC1-PC8 group HDMI USB input |
| 12 | PC1, PC2，PC3，PC4，PC5，PC6，PC7，PC8， switch buttons and status indicators |
| 13 | USB keyboard mouse connectors |
| 14 | USB2.0 connectors |

6.Diagram

7. Warranty

Pease keep the user manual in a place where safety and easy to find so that you can read it immediately when meet problems.

safety instruction:

- Please do not disassemble this product by yourself because it doesn't including DIY function.

- Please don't make the product touch water in case damaged, because it is Not waterproof design.

- Please use dry cloth material when clean the product surface.

- Please do not use this product in the environment of too wet or too high temperature.

- Please avoid contact with corrosive substances.

- When there is damage or other physical damage to the product, please stop using and contact with customer service to get support.