HDMI 1*4 SPLITTER Extender

USER MANUAL

1.Description

The HDMI extender provides extension of HDMI signals long distances over a single CAT6 cable, transform one channel input to 4 channels output, it supports high resolution up to 1080p. The extender can use for a wide range of applications requiring long distance transmission of high resolution with high quality by its good stability and powerful security.

2.Features

- Transmits 1 HDMI video and audio signals up to 50M over a single CAT6 cable;

- Support video resolution up to 1920*1080@60Hz, 3D signal;

- Transform one channel HDMI input to 4 channels Cat6 signal output;

- Support EDID copy function, can match many kinds of display device;

- Support POC function;

- Compliance with HDMI 1.3 and HDCP 1.2 standard;

- High compatibility, can auto-match source and display device;

- Built-in automatic adjustment system, make the image smooth, clear and stable;

- Easy to install, plug and play;

3.Package list

        - HDMI Transmitter ……………………1 pcs

        - HDMI Receiver........................ …4 pcs

        - Power supply …………………….….. 1 pcs

- User manual …………………………. 1 pcs

        - Foot Pad…………………………………..4 pcs

4.Specifications

| Parameter |  | Description |
| --- | --- | --- |
| Video | Standards | HDMI 1.3; HDCP 1.2 |
|  | Maximum pixel clock | 165MHz |
|  | Maximum data rate | 6.75Gbps |
|  | Resolution | 1920X1080@60Hz |
|  | Connector | HDMI-A |
|  | Impedance | 100Ω |
| Ethernet | Interface | RJ45 |
|  | Transmission distance | CAT6---50M |
| Other | Power supply | The power adapter: DC 12V/1A |
|  | Power dissipation | MAX 8W |
|  | Temperature | Operating: -5℃ ~ +70℃ |
|  | Humidity | Operating: 5% ~ 90% |
|  | Dimension | TX:150*71*23mm/RX:81*45*23mm |
|  | The warranty | 1 year free warranty |

5.Panel

Transmitter

Receiver

| Port name | Description |
| --- | --- |
| HDMI IN/ HDMI OUT | HDMI signal input/ output |
| UTP | RJ45 network interface |
| EDID | ①.Plug the power supply into the "DC 12V" port of the transmitter . ②. Use a HDMI cable connect the monitor to the transmitter. ③. Long press the EDID button(On the transmitter ) until the STA light flashes, and then the flashing stops. Completed copy EDID. |
| DC 12V | Power adapter socket |
| LED indicator | Description |
| STATUS/ STA | Video signal indicator |
| POWER/ PWR | System power indicator |

6.Installation

- Connect the provided DC 12V power supplies to the power socket of the transmitter and the receiver,

- Connect an HDMI cable between the HDMI input port of transmitter and the HDMI output port of the video source,

- Connect the HDMI output port of receiver to the display device with HDMI cable,

- Connect the transmitter RJ45 port to the receiver RJ45 port by a single CAT6 cable.

7.Diagram

- Warranty

Please keep the user manual in a place where safety and easy to find so that you can read it immediately when meet problems.

Safety instruction:

- Please do not disassemble this product by yourself because it doesn't including DIY function.

- Please don't make the product touch water in case damaged, because it is Not waterproof design.

- Please use dry cloth material when clean the product surface.

- Please do not use this product in the environment of too wet or too high temperature.

- Please avoid contact with corrosive substances.

- When there is damage or other physical damage to the product, please stop using and contact with customer service to get support.